Simplified Arithmetic Hilbert Transform

based Wide-Band Real-Time

Digital Frequency Estimator

Jean-Paul Sandoz

Professor of Electronics and Signal Processing

Part 1	Introduction

Part 2	Basic Theoretical Analysis

Part 3	Implementation

Part 4	Simulation Results

Part 5	Conclusions

Part 1   -   Introduction

Objectives of this work

- To develop a practical solution to FPGA/CPLD implementation of a Hilbert Transform (HT) based Instantaneous Frequency Estimator.

- Multiplication-free structure

Most significant Innovations

- Substitution of the Arctan function by the One-Bit based balanced Quadri-Correlator.

- Up Frequency Conversion and Single-Side Band generation at IF to achieve a wide Frequency Range of operation.

Key Advantages

- Maintain good performances in Impulsive Noise environment and/or Variable SNR

- Low Power consumption designs or High Frequency applications.

Important characteristics

- Frequency range of operation:  200 Hz  4 kHz

- Low system clock frequency: 80 kHz

- Tracking error standard deviation of slow varying frequency:  3 Hz

- Group delay:  3 ms

- 200 Hz to 4 kHz transient time (10%  90%): 2.5 ms

- Maximum to minimum frequencies of operation ratio:  20

- Resistant to wide-band additive background noise

- Very robust to impulsive noise

- Fully controllable frequency estimation accuracy and response time

Principal blocs constituting this design

- Multiplication-free all-pass digital filter Hilbert Transformer

- Multiplication-free up-frequency converter

- Single–side band generation at intermediate frequency

(IF = 6  9.8 kHz)

- “One-Bit” based balanced Quadri-Correlator” operating at IF

- Running Averagers controlling the frequency estimator

dynamic performances

Part 2   -   Basic Theoretical Analysis

Bloc diagram of the complete structure

- xin(t) = A(t)sin[2 freqxin(t) t]  : Input signal

freqxin(t) = Instantaneous frequency , 200 Hz   freqxin  4000 Hz

- The Hilbert Transformer output is an analytic signal xHT(t) :

~

~

xHT(t) = xin(t) + j xin(t)   with   xin(t)  = H [xin(t)] = xin(t) (1 /t)

- The square-wave generator and the delay element produce a complex carrier with a fundamental frequency freqSQ of 10kHz

- The multiplier outputs are combined such that the lower side-band produced is an analytic signal :

LSB-In, LSB-Q

- The multiple harmonics generated by the square signal and present on this analytic signal are attenuated by low-pass filters.

Analytic signal instantaneous frequency :       freqSQ – freqxin(t)

Hilbert Transform properties :

x(t) : Real function of time



~



~

~

f(t) = (t) / 2 : Instantaneous frequency

A(t) : Instantaneous envelope

Under certain conditions, One-Bit” balanced Quadri-Correlator determines the  instantaneous frequency  of its input signals !!

Duty cycle

proportional to

In signal freq.

Bloc diagram of the  One-Bit” balanced Quadri-Correlator

Part 3   -   Implementation

- Hilbert transformer

Three second-order all-pass filters and one unit delay element

.

Bloc diagram of the Hilbert Transformer

Where :

Numerical Optimization (i.e. coefficient round-off)

Ck =  2-m  2-n

( m  3 and n  4 , positive integers )

  Multiplication Free Implementation

## Complex multiplier

- First inputs: Square-wave generator (i.e. +1 and –1) and its delayed version

- Second inputs: Hilbert transformer outputs.

 The implementation :  two’s complement functions

## Lower side-band selector

- Realized by Phasing Technique from complex multiplier outputs.

- Strongest unwanted frequencies : Attenuated by at least 10 dB

8 kHz	 16 kHz	  24 kHz	  32 kHz	  40 kHz

40 dB

From

Harm. 3

From

Harm. 5

Lower side-band output spectrum

## Low-Pass Filter (realized with two identical averagers)

40 dB

Lower side-band + Low-pass filter output spectrum

Part 4   -   Simulation Results

- Two Tones Test

Top: xin(t)  300 Hz –3.9 kHz,  Bottom: Instant. Freq. Estimation

- Fast varying Frequency

Input Signal Freq. (dotted) and Estimated Freq. (continuous)

g (group delay) = 3 ms  ;  trans (10%  90%) = 2.5 ms

(Slow varying frequency tracking error: 3 Hz RMS)

- Sinusoidal Signal + Band-Limited Gaussian Noise

Top: 	Sinusoidal signal  +  Noise   (SNR =  10 dB)

Bottom: 	Instantaneous Frequency Estimation

- Two Sinusoidal Signals with varying Frequencies

Top: 	 Input signal (Signal1  is 6dB stronger than Signal2)

Bottom: Recovered Instantaneous Frequency of Signal1

- Sinusoidal Signal and Impulsive Noise

Top: Sine-wave + Impulsive noise;  Bottom: Instant. Freq. Esti.

The original One-Bit signal processing technique implementation  of the Balanced Quadri-Correlator provides a design that is inherently robust to impulsive noise and/or abrupt amplitude changes of the input signal  !!

- Instantaneous Envelope and Frequency Estimation

Instantaneous Envelope Estimation

Instantaneous Frequency Estimation

Top:  Amplitude and Frequency modulated Signal

Center and Bottom: Instantaneous Ampl. and Freq.  Estimation

Part 5   -   Conclusions

The first series of computer simulations (SystemView) of the proposed new scheme have shown that:

- Its performances are very good in Non-Stationary Environment

- 100% multiplication-free

- Very good dynamic performances

- Overall performances match up the  Arctan   function solution

- Very robust against impulsive and/or burst type noise

- Instantaneous envelope estimation without complex computations

- Low power consumption designs or high speed applications

Current Fields of Investigation

- Dynamic speed measurement of very small electrical motors

- Instantaneous Frequency Estimation of Time-Decaying Processes

- Ultra-Sound based Instantaneous Velocity Measurement (Doppler)

Potential Applications

- Instant. Freq. Pattern Analysis in Ultrasonic Structural Defect Assessment

- Real-Time Vibration Analysis of Dynamic Systems

- Time-Varying characteristics applied to Diagnostics

Your Brain before this presentation:

Hilbert Transform

Quadri-Correlator

Lower side-band selector

Complex Multiplier

Your Brain after this presentation:

Hilbert Transform

Quadri-Correlator

Lower side-band selector

Complex Multiplier